Vision and Voice Guided Robotic Arm

Task 2 Engineering Log — CoppeliaSim Arm Skeleton

Digital Prototype • 5-DOF Arm • Manual Joint Validation

| Task objective | Build a custom digital robotic arm skeleton in CoppeliaSim and verify each revolute joint manually before Python control. |
| --- | --- |
| Final result | A working 5-DOF serial chain: shoulder yaw, shoulder pitch, elbow flexion, wrist roll, and wrist pitch, ending in a simple palm block. |
| Main software | CoppeliaSim Edu |

# 1. Project File and Naming Convention

Main CoppeliaSim scene: simulation/vision_voice_robot_arm.ttt

The scene was saved inside the GitHub repository rather than in a random local folder so the simulation becomes part of the project history and can be versioned together with code and documentation.

Naming convention: lowercase snake_case for every custom object and joint.

Purpose: Python will later retrieve objects by name, so descriptive and stable aliases reduce control-code ambiguity.

| Object | Role |
| --- | --- |
| robot_base | Fixed foundation of the robot. |
| joint_shoulder_yaw | Rotates the shoulder assembly around the vertical axis. |
| shoulder_platform | Intermediate link carried by shoulder yaw. |
| joint_shoulder_pitch | Raises/lowers the upper arm in a single plane. |
| link_upper_arm | Upper-arm rigid link. |
| joint_elbow | Bends the forearm relative to the upper arm. |
| link_forearm | Forearm rigid link. |
| joint_wrist_roll | Twists the complete wrist/hand around the forearm axis. |
| joint_wrist_pitch | Bends the palm relative to the forearm. |
| hand_palm | Simple placeholder palm used to validate wrist motion. |

# 2. Final Kinematic Chain

Final hierarchy:

robot_base

└── joint_shoulder_yaw

└── shoulder_platform

└── joint_shoulder_pitch

└── link_upper_arm

└── joint_elbow

└── link_forearm

└── joint_wrist_roll

└── joint_wrist_pitch

└── hand_palm

Why hierarchy matters: a downstream child must inherit every upstream transformation. For example, yawing the shoulder must rotate the shoulder-pitch joint, upper arm, elbow, forearm, both wrist joints, and palm together.

# 3. Geometry and Prototype Dimensions

| Component | Prototype value | Reason |
| --- | --- | --- |
| robot_base | 0.20 × 0.20 × 0.04 m | Large simple base for stable visualization; center at Z = 0.02 m places its bottom on the floor. |
| shoulder_platform | 0.07 × 0.07 × 0.04 m | Compact intermediate link between yaw and pitch axes. |
| link_upper_arm | 0.03 × 0.03 × 0.18 m | Long enough to clearly visualize shoulder and elbow motion while remaining easy to position. |
| link_forearm | 0.03 × 0.03 × 0.16 m | Slightly shorter than upper arm; adequate for a clear elbow test. |
| hand_palm | 0.07 × 0.02 × 0.09 m | Simple placeholder for wrist tests; fingers will be added later. |
| Typical joint visual length | ~0.04–0.05 m | Shortened from the default because long cylinders made the scene difficult to read. |
| Typical joint visual diameter | ~0.015–0.02 m | Large enough to see axes clearly without dominating the model. |

# 4. Joint Orientation Decisions

A CoppeliaSim revolute joint rotates around its local Z-axis. Therefore, the object orientation of each joint determines the physical hinge axis. This mounting orientation is different from the joint Position [deg], which is the commanded movement angle.

| Joint | Mounting axis / orientation | Why | Result |
| --- | --- | --- | --- |
| Shoulder yaw | Vertical axis (default-like orientation) | Yaw must turn the whole arm left/right around the vertical base axis. | Rotates all downstream links around the base. |
| Shoulder pitch | Horizontal axis; approximately X = 90°, Y = 0°, Z = 0° during setup | Pitch needs a hinge perpendicular to the long arm direction so the upper arm swings in one plane. | Raises/lowers the upper arm. |
| Elbow | Horizontal and parallel to shoulder pitch | The elbow bends the forearm in a similar hinge plane to shoulder pitch. | Forearm bends relative to upper arm. |
| Wrist pitch | Horizontal and parallel to elbow | The palm needs an up/down bending motion. | Palm bends relative to forearm. |
| Wrist roll | Axis aligned with the forearm length | Roll is not another hinge; it must twist the entire wrist/hand around the forearm axis. | Pitch joint + palm rotate around forearm axis. |

# 5. Test Angles and Why They Were Chosen

The angles used in Task 2 are debugging angles, not final anatomical limits. They were chosen because they are large enough to make movement obvious while remaining simple to reason about.

| Joint / test | Angles used | Reason |
| --- | --- | --- |
| Shoulder yaw | 0°, +45°, +90°, −45° | 45° is especially useful because a square shoulder platform can look unchanged after a 90° rotation; 45° makes rotation visually unmistakable. |
| Shoulder pitch | 0°, +30°, +60°, −30° | 30° gives a safe visible change; 60° verifies the motion continues correctly; −30° checks the opposite direction. |
| Elbow | 0°, +30°, +60°, +90° | Progressively confirms independent bending and reaches a clearly recognizable right-angle elbow pose. |
| Wrist pitch | 0°, +30°, −30° | Enough to verify palm bending in both directions without confusing the scene. |
| Wrist roll | 0°, +30°, +60°, −30° | 30° confirms twist direction, 60° confirms continuous roll behavior, and −30° verifies reverse rotation. |

# 6. Issues Encountered and How They Were Solved

| Issue / symptom | Root cause | Fix | Engineering lesson |
| --- | --- | --- | --- |
| Shoulder yaw appeared not to rotate. | The square shoulder platform was centered on the joint, so some rotations were visually difficult to notice. | Temporarily offset the platform and test at 45°, then return it to the center. | A mechanism can be working even when the geometry makes the motion visually ambiguous; use asymmetric test geometry or a non-symmetric angle. |
| Confusion between Object Orientation and Joint Position. | Object Rotation/Orientation changes how the hinge is mounted; Joint Position [deg] commands movement around that mounted axis. | Use Object Orientation only during setup; after mounting, test motion only through Joint Position [deg]. | Separate mechanical frame definition from actuation state. |
| Shoulder pitch could move only in one plane. | A single revolute joint has exactly 1 DOF. | Combine shoulder pitch with upstream shoulder yaw. Yaw rotates the pitch plane, creating 2-DOF shoulder motion. | Multi-axis motion is produced by serial joints, not by forcing one revolute joint to move in multiple planes. |
| Second joint became displaced/diagonal while testing. | The entire joint object was repeatedly rotated instead of changing its kinematic position. | Undo/re-center the object, fix its mounting orientation once, then use Joint Position [deg]. | Do not use mounting transforms as movement commands. |
| Elbow joint was initially too long and positioned alongside the upper arm. | Default joint visual length and placement made the hinge axis unclear. | Shorten to ~0.05 m, place its center at the top end of the upper arm, and orient it horizontally. | Visual scale matters for debugging even when it does not affect kinematics. |
| Wrist pitch appeared diagonal. | Its mounting axis was not parallel to the elbow hinge. | Set object orientation so the wrist-pitch axis is horizontal and parallel to elbow axis. | Compare axes visually across the kinematic chain. |
| Need for wrist roll discovered after wrist pitch worked. | Pitch alone cannot reproduce hand orientation; it only bends in one plane. | Insert joint_wrist_roll between forearm and wrist pitch with its axis aligned to the forearm. | Different human wrist motions require orthogonal rotational DOFs. |
| Powerful chain moved in unexpected directions during testing. | Upstream joints transform every downstream frame, so later joints no longer align with world axes after yaw/pitch changes. | Return all joints to 0° before geometry edits; use parent/local frames for placement when possible. | Serial-chain debugging is easiest from a known neutral configuration. |

# 7. Important Concepts Learned in Task 2

A revolute joint contributes one independent rotational degree of freedom.

Object hierarchy defines which links inherit a joint transformation.

World coordinates are useful for initial global placement; parent-frame coordinates are usually better for serial-chain assembly.

A link attached to a joint should usually be offset by half its own length so one end begins at the joint center.

Joint object orientation defines the axis; Joint Position [deg] defines motion about that axis.

Neutral pose (all joint positions = 0°) should be restored before adding or repositioning downstream components.

Visual joint dimensions do not change kinematics, but clean visualization makes debugging much easier.

Test angles are validation tools; realistic anatomical limits will be chosen later during camera mapping and physical design.

# 8. Task 2 Completion Test

Task 2 is considered complete because each joint was manually validated in CoppeliaSim and the serial hierarchy behaved correctly.

| Validation | Expected behavior | Result |
| --- | --- | --- |
| Shoulder yaw | Entire downstream arm rotates around vertical base axis. | PASS |
| Shoulder pitch | Upper arm and everything after it swing in the pitch plane. | PASS |
| Elbow | Forearm and wrist move relative to upper arm. | PASS |
| Wrist roll | Wrist-pitch joint and palm twist around forearm axis. | PASS |
| Wrist pitch | Palm bends relative to the forearm/wrist-roll assembly. | PASS |
| Hierarchy | Every downstream child follows all upstream joints without detaching. | PASS |

# 9. Next Task

Task 3 — Python → CoppeliaSim Control. The next objective is to control these same five joints from Python, proving the software can command the simulator before camera tracking is introduced.